YERSINIA LAYER 2 ATTACK & DEFENSE ANALYSIS

Yersinia is a widely-used framework in the field of cybersecurity for penetration testing, vulnerability assessment, and Layer 2 auditing. It helps security professionals identify and address network infrastructure weaknesses in protocols and services.

It exploits vulnerabilities in Layer 2 protocols including Spanning Tree Protocol (STP), Cisco Discovery Protocol (CDP), Dynamic Host Configuration Protocol (DHCP), Hot Standby Router Protocol (HSRP), and Dynamic Trunking Protocol (DTP).

A DHCP starvation attack is a Denial-of-Service (DoS) technique where an adversary floods a DHCP server with fake DHCPDISCOVER messages using spoofed MAC addresses to completely exhaust the available IP address pool.

1. Topology Setup & Baseline Switch Configuration

In this topology, VLAN 10 (CORPORATE) was configured to segment the broadcast domain, ensuring the legitimate corporate DHCP server distributes IP addresses exclusively to authorized hosts on that VLAN.

2. DHCP Pool Configuration (Corporate vs. Rogue)
Configure the corporate DHCP pool (CORP-DHCP) for the network according to the preferred corporate subnet (192.168.10.0/24):

Configure the rogue DHCP server with an unauthorized IP scope (10.0.0.0/24):

Baseline test: Legitimate DHCP request succeeded from CORP-DHCP (192.168.10.13 assigned):

3. Threat Modeling: Rogue DHCP & Starvation
Because the attacker deployed an unauthorized DHCP server on the same broadcast domain, client devices can inadvertently obtain lease parameters from ROGUE-DHCP instead of CORP-DHCP.

If the legitimate CORP-DHCP server becomes unavailable—or if its address pool is exhausted through attacks such as:

- • DHCP Starvation (pool exhaustion)

- • DHCPNAK injection (targeted session disruption)

When legitimate distribution fails, the client broadcasts a new DHCPDISCOVER packet. The rogue server replies faster with an offer, assigning the client an address in the 10.0.0.0/24 subnet and establishing an unauthorized default gateway to stage a Man-in-the-Middle (MitM) attack:

4. Implementing DHCP Snooping and Port Security

Globally enable DHCP snooping and establish the trust boundary on the legitimate server uplink:

Trust Boundary: Setting interface FastEthernet0/24 to 'trusted' allows the legitimate corporate DHCP server to send response messages (DHCPOFFER, DHCPACK) into the network. Untrusted interfaces will automatically drop unauthorized server replies.

Rate Limiting: Applying 'limit rate 5' restricts untrusted access interfaces to 5 DHCP packets per second, thwarting high-speed starvation floods.

MAC Address Spoofing & Port Security Mitigation

MAC address spoofing allows an adversary to impersonate valid hosts, intercept traffic, or stage Layer 2 MitM and starvation attacks.

Objective: Lock down untrusted access ports to prevent MAC address spoofing using Cisco Port Security:

- • Maximum MAC Limit: Restricts each access interface to a maximum of 2 learned MAC addresses.

- • Violation Action (restrict): Drops frames with unauthorized source MAC addresses, logs the violation, and generates an alert without disabling the physical port.

- • Sticky Learning: Dynamically records learned source MAC addresses and converts them into persistent entries within the running configuration.

Simulation Verification: In Packet Tracer simulation mode, the rogue DHCP server attempts to service the request from ADMIN-PC by returning a DHCPOFFER packet.

Mitigation Result: Because DHCP snooping is active, the switch identifies the untrusted access port sourcing a server response and immediately discards the rogue DHCPOFFER packet at Layer 2.

5. Forensic Packet Capture on Lab VM

Live validation of the attack was conducted inside Kali Linux to analyze packet-level behavior.

To capture authentic Layer 2 forensic evidence, Yersinia was launched against the local network interface to simulate a live DHCP starvation flood.

Launch Wireshark and begin sniffing traffic on the eth0 interface:

Set the Wireshark display filter to: dhcp (or legacy bootp)

The 'bootp' display filter captures all DHCP traffic because DHCP is an extension of the Bootstrap Protocol (BOOTP).

Note: In modern versions of Wireshark, 'dhcp' is standard and preferred over 'bootp'.

Execute the DHCP starvation attack from Kali Linux using Yersinia:

sudo yersinia dhcp -attack 1 -interface eth0

Terminal execution confirmed Yersinia actively broadcasting high-frequency discover frames. Transition to Wireshark to observe live packet activity:

Traffic Observation: Yersinia transmits a rapid deluge of DHCPDISCOVER frames. Although the server attempts to issue DHCPOFFER replies, the transactions remain incomplete (no final DHCPACK), keeping the server's allocation pool in a persistent half-open state until capacity is depleted.

Frame & Payload Dissection

Comparing the frames reveals that the source MAC address rotates with each packet (e.g., transitioning from 0d:19:13:12:1a:18 to 52:55:0a:00:02:02). The destination IP address remains the limited broadcast (255.255.255.255).

Crucially, Yersinia randomizes the 'Client MAC address' (chaddr) field within the DHCP payload. Because DHCP server leases are keyed to the client identifier and hardware address, varying this field forces the server to bind new IP addresses to each spoofed MAC until the entire scope is starved.

6. Remediation & Defense Summary

Implementing DHCP Snooping (trust boundaries and rate limiting) alongside Cisco Port Security (sticky MAC learning and violation restriction) effectively mitigates DHCP starvation and rogue server attacks by dropping unauthorized offers and blocking excessive spoofed MAC addresses at the access layer.